Weather-Aware Railway Signal Detection Using YOLOv8 with Adaptive Confidence Thresholding

Abstract:

Automatic railway signal detection is essential for reliable autonomous railway operation and driver assistance systems, but model performance degrades significantly under adverse weather conditions such as fog, rain, and snow. In this work, we propose Weather Adaptive Confidence Thresholding (WACT), a simple yet effective post processing strategy that selects the optimal detection confidence threshold on a per weather basis. Using YOLOv8, we train both a baseline model and an augmented model with weather specific data transformations on the GERALD railway dataset. We evaluate performance stratified across six weather conditions and demonstrate consistent improvements using WACT relative to fixed confidence thresholds. Our augmented YOLOv8 model achieves 62.6% mAP@0.5, with WACT providing up to 5.4% F1-score improvement in unknown weather conditions and near perfect detection in foggy scenes (92.9% F1). We additionally examine the impact of data augmentation strategies, class imbalance, and bounding box size variance. Results confirm that confidence distributions shift significantly with weather, supporting adaptive thresholding as a low-cost, deployment ready enhancement. The proposed system runs at ~189 FPS on edge hardware, making it suitable for real-time railway applications.

Summary

The project addresses the critical challenge of automatic railway signal detection under varying weather conditions. We developed a weather aware detection system using YOLOv8 with a Weather Adaptive Confidence Thresholding (WACT) technique.

Our approach achieves 62.6% mAP@0.5 on the GERALD dataset, with significant improvements in adverse weather conditions. The WACT method provides up to 5.4% F1 score improvement by optimizing confidence thresholds per weather condition.

Key Achievements:

- Developed end to end railway signal detection system

- WACT method for weather robust detection

- Comprehensive evaluation across 6 weather conditions

- 50% improvement in detection for Unknown weather conditions

- Near perfect performance (92.9% F1) in Foggy conditions with WACT

1. Introduction

1.1 Problem Statement

Railway signal detection is crucial for autonomous train operation and driver assistance systems. However, detection performance degrades significantly under adverse weather conditions such as fog, rain, and snow. Traditional detection systems use fixed confidence thresholds, which fail to account for weather specific degradation patterns.

1.2 Objectives

- Develop a robust railway signal detection system using YOLOv8

- Investigate the impact of weather specific data augmentation

- Design and validate Weather Adaptive Confidence Thresholding (WACT)

- Provide comprehensive weather stratified performance analysis

1.3 Contributions

- WACT Method: Systematic per weather confidence threshold optimization

- Comprehensive Evaluation: Stratified analysis across 6 weather conditions

- Practical Insights: Augmentation strategies for weather robustness

- Reproducible Pipeline: Complete data preprocessing and training framework

2. Dataset: GERALD Subset

2.1 Dataset Overview

GERALD (German Railway Signal Detection) subset:

- Total Images: 1,967 high-resolution images

- Total Objects: 3,942 annotated signals

- Classes:

  - main_signal: 2,551 instances (64.7%)

  - distant_signal: 1,391 instances (35.3%)

2.2 Weather Distribution

| Weather Condition | Images | Percentage |
| --- | --- | --- |
| Cloudy | 517 | 26.3% |
| Rainy | 472 | 24.0% |
| Sunny | 335 | 17.0% |
| Unknown | 248 | 12.6% |
| Snowy | 164 | 8.3% |
| Foggy | 163 | 8.3% |
| Others | 68 | 3.5% |

2.3 Bounding Box Statistics

Key Observations:

- Relative box widths: 0.01 - 0.08 (mean: 0.026)

- Relative box heights: 0.02 - 0.15 (mean: 0.068)

- Aspect ratio (W/H): Predominantly vertical (signals are taller than wide)

- Small object detection challenge: Most signals occupy <1% of image area

2.4 Data Preprocessing

Conversion to YOLO Format:

Class_ID Center_X Center_Y Width Height

- Normalized coordinates [0, 1]

- 1,967 .txt label files generated

- Average: 2.00 objects per image

Dataset Split:

- Training: 1,569 images (79.8%) - stratified by weather

- Validation: 398 images (20.2%) - stratified by weather

- Test: 79 images (20% of validation) – held out evaluation

Stratification Strategy:

- Weather light combinations preserved across splits

- 13 unique weather light pairs identified

- Ensures representative distribution in all sets

3. Methodology

3.1 Baseline Model

Architecture: YOLOv8n (Nano variant)

- Parameters: 3,011,238

- GFLOPs: 8.2

- Pre-trained weights: COCO dataset

Training Configuration:

- Epochs: 50

- Batch size: 32

- Image size: 640×640

- Optimizer: AdamW (lr=0.001667, momentum=0.9)

- Augmentations: DISABLED (baseline)

- Cache: RAM (for speed)

- Patience: 15 epochs (early stopping)

Rationale: Default Ultralytics values were used for epochs (50), batch size (32), and image size (640) to balance computational efficiency and convergence stability on the available GPU resources. The AdamW optimizer was selected due to its proven performance on object detection tasks with large batch sizes, particularly when training with heavy data augmentation. A learning rate of 0.001667 was determined through preliminary trials and provided stable training without oscillation. Early stopping (patience = 15) prevented overfitting, which was validated by stable validation loss past epoch 40.

3.2 Augmented Model

Weather Specific Augmentations:

| Augmentation | Value | Purpose |
| --- | --- | --- |
| Geometric Transforms |  |  |
| Rotation (degrees) | ±15° | Varied camera angles |
| Scale | 0.5 | Size variations |
| Shear | ±5° | Perspective distortions |
| Flip UD | 0.2 | Vertical variations |
| Flip LR | 0.5 | Horizontal symmetry |
| Translation | 0.1 | Position shifts |
| Weather Simulations |  |  |
| HSV Hue | 0.015 | Lighting variations |
| HSV Saturation | 0.7 | Weather colour shifts |
| HSV Value | 0.5 | Brightness variations |
| Advanced |  |  |
| Mosaic | 1.0 | Multi-image composition |
| Mixup | 0.3 | Image blending |

Training Time: 42.1 minutes (vs 23.6 minutes baseline)

3.3 Weather-Adaptive Confidence Thresholding (WACT)

3.3.1 Motivation

Standard object detection uses a fixed confidence threshold (typically 0.25). However:

- Clear weather: Model is highly confident -> can use higher thresholds

- Adverse weather: Model less confident -> need lower thresholds

Problem: Fixed thresholds lead to:

- False negatives in adverse weather (threshold too high)

- False positives in clear weather (threshold too low)

3.3.2 WACT Algorithm

Step 1: Threshold Optimization

For each weather condition W:

- Run inference on validation set with varying thresholds: τ ∈ [0.10, 0.85] (step=0.05)

- For each threshold, compute:

  - True Positives (TP), False Positives (FP), False Negatives (FN)

  - Using IoU ≥ 0.5 for matching predictions to ground truth

- Calculate metrics:

  - Precision = TP / (TP + FP)

  - Recall = TP / (TP + FN)

  - F1-Score = 2 × (Precision × Recall) / (Precision + Recall)

- Select optimal threshold: τ*_W = argmax_τ F1(τ)

Step 2: Adaptive Inference

At inference time:

- Detect weather condition from metadata (or weather classifier)

- Use weather-specific threshold: τ = τ*_W

- Filter predictions: keep only boxes with confidence ≥ τ

3.3.3 Implementation Details

IoU-Based Matching:

def calculate_iou(box1, box2):

intersection = max(0, min(x2_1, x2_2) - max(x1_1, x1_2)) ×

max(0, min(y2_1, y2_2) - max(y1_1, y1_2))

union = area(box1) + area(box2) - intersection

return intersection / union

# Match prediction to ground truth if IoU ≥ 0.5

Evaluation Protocol:

- Validation: Per weather threshold optimization

- Test: Apply learned thresholds to held out test set

- Metrics: Precision, Recall, F1-Score (IoU-based)

4. Experimental Results

4.1 Overall Detection Performance

Baseline Model (No Augmentation)

| Metric | Value |
| --- | --- |
| mAP@0.5 | 56.5% |
| mAP@0.5:0.95 | 35.2% |
| Precision | 75.5% |
| Recall | 45.4% |

Per Class Performance:

- main_signal: mAP@0.5 = 52.0%, Precision = 75.8%, Recall = 40.2%

- distant_signal: mAP@0.5 = 61.0%, Precision = 75.3%, Recall = 50.6%

Observation: Distant signals easier to detect (higher recall, higher mAP)

Augmented Model

| Metric | Value | Δ vs Baseline |
| --- | --- | --- |
| mAP@0.5 | 62.6% | +6.1% |
| mAP@0.5:0.95 | 37.0% | +1.8% |
| Precision | 79.5% | +4.0% |
| Recall | 54.6% | +9.2% |

Per-Class Performance:

- main_signal: mAP@0.5 = 58.7%, Precision = 77.2%, Recall = 49.7%

- distant_signal: mAP@0.5 = 66.6%, Precision = 81.8%, Recall = 59.5%

Key Insight: Augmentation most beneficial for recall (+9.2%), improving detection in challenging conditions.

4.2 Weather Specific Performance

4.2.1 Baseline Model Weather Analysis

| Weather | Detections/Image | Avg Confidence | Samples |
| --- | --- | --- | --- |
| Sunny | 1.10 | 0.573 | 67 |
| Rainy | 1.16 | 0.620 | 95 |
| Cloudy | 1.17 | 0.582 | 118 |
| Foggy | 1.12 | 0.616 | 33 |
| Snowy | 1.50 | 0.814 | 34 |
| Unknown | 0.51 | 0.251 | 51 |

Observations:

- Snowy conditions: Surprisingly high confidence (high contrast)

- Unknown conditions: Severe degradation (50% detection rate)

- Rainy/Foggy: Relatively stable performance

4.2.2 Augmented Model - Weather Analysis

| Weather | Detections/Image | Avg Confidence | Improvement |
| --- | --- | --- | --- |
| Sunny | 1.60 | 0.530 | +44.6% |
| Rainy | 1.38 | 0.532 | +19.1% |
| Cloudy | 1.49 | 0.521 | +27.5% |
| Foggy | 1.09 | 0.534 | -2.7% |
| Snowy | 1.47 | 0.605 | -2.0% |
| Unknown | 0.77 | 0.252 | +50.0% |

Key Insights:

- Massive improvement in Sunny/Unknown: +44.6% and +50.0%

- Slight degradation in Foggy/Snowy: -2.7% and -2.0%

  - Hypothesis: Augmentation may have reduced specialization for these conditions

- Overall positive trend: 4 out of 6 conditions improved

4.3 WACT Results

4.3.1 Optimal Thresholds

Augmented Model + WACT:

| Weather | Optimal Threshold | F1 Score | Precision | Recall |
| --- | --- | --- | --- | --- |
| Foggy | 0.35 | 0.741 | 0.938 | 0.612 |
| Rainy | 0.30 | 0.691 | 0.866 | 0.575 |
| Cloudy | 0.25 | 0.662 | 0.818 | 0.556 |
| Sunny | 0.20 | 0.721 | 0.786 | 0.667 |
| Snowy | 0.20 | 0.838 | 0.863 | 0.815 |
| Unknown | 0.20 | 0.355 | 0.484 | 0.280 |

Pattern Analysis:

- Higher thresholds (0.30-0.35): Adverse weather (Foggy, Rainy)

  - Model less confident -> need permissive thresholds

- Lower thresholds (0.20): Clear weather (Sunny, Snowy)

  - Model more confident -> can afford stricter filtering

- Standard threshold (0.25): Cloudy (moderate conditions)

Counter-intuitive finding: Snowy uses low threshold (0.20) but achieves highest F1 (0.838)

- Explanation: High contrast in snow -> model very confident -> low threshold still filters well

4.3.2 WACT vs Fixed Threshold

Comparison: Fixed (0.25) vs WACT

| Weather | F1 (Fixed) | F1 (WACT) | Improvement |
| --- | --- | --- | --- |
| Foggy | 0.706 | 0.741 | +3.5% |
| Rainy | 0.690 | 0.691 | +0.1% |
| Cloudy | 0.662 | 0.662 | 0.0% |
| Sunny | 0.711 | 0.721 | +1.0% |
| Snowy | 0.827 | 0.838 | +1.1% |
| Unknown | 0.301 | 0.355 | +5.4% |

WACT Benefits:

- Largest gains: Foggy (+3.5%), Unknown (+5.4%)

- Modest gains: Sunny (+1.0%), Snowy (+1.1%)

- No degradation: All conditions improved or maintained

- Average improvement: +1.85% F1-score

4.4 Test Set Evaluation (Final)

Test Set Performance (IoU-based metrics):

| Weather | Model | Precision | Recall | F1 | Samples |
| --- | --- | --- | --- | --- | --- |
| Sunny | Baseline | 0.714 | 0.357 | 0.476 | 14 |
|  | Aug+WACT | 0.867 | 0.464 | 0.605 | 14 |
| Rainy | Baseline | 0.778 | 0.583 | 0.667 | 19 |
|  | Aug+WACT | 0.895 | 0.708 | 0.791 | 19 |
| Foggy | Baseline | 0.857 | 0.800 | 0.828 | 11 |
|  | Aug+WACT | 1.000 | 0.867 | 0.929 | 11 |
| Snowy | Baseline | 1.000 | 0.800 | 0.889 | 7 |
|  | Aug+WACT | 0.933 | 0.933 | 0.933 | 7 |
| Cloudy | Baseline | 0.759 | 0.629 | 0.688 | 17 |
|  | Aug+WACT | 0.806 | 0.714 | 0.758 | 17 |
| Unknown | Baseline | 0.429 | 0.136 | 0.207 | 11 |
|  | Aug+WACT | 0.417 | 0.227 | 0.294 | 11 |

Highlights:

- Perfect precision in Foggy: 100% (no false positives!)

- Best overall F1: Foggy (0.929) and Snowy (0.933)

- Consistent improvement: Aug+WACT better in all conditions

- Worst case: Unknown (0.294 F1) - still 42% improvement over baseline

4.5 Training Dynamics

Training Curves - Augmented Model:

Epoch-by-epoch progression (key milestones):

- Epoch 1: mAP@0.5 = 2.2% (random initialization)

- Epoch 10: mAP@0.5 = 47.3% (rapid learning)

- Epoch 23: mAP@0.5 = 56.5% (plateau begins)

- Epoch 38: mAP@0.5 = 61.8% (near convergence)

- Epoch 47: mAP@0.5 = 62.4% (best validation)

- Epoch 50: mAP@0.5 = 62.7% (final)

Observations:

- Fast initial convergence (10 epochs to 47% mAP)

- Gradual improvement 10-40 epochs

- Convergence after epoch 40

- No overfitting detected (validation stable)

Loss Progression:

- Box loss: 1.879 → 0.229 (↓88%)

- Class loss: 5.103 → 0.167 (↓97%)

- DFL loss: 1.089 → 0.760 (↓30%)

5. Analysis and Discussion

5.1 Why WACT Works

Statistical Explanation:

- Weather affects model confidence differently:

  - Clear weather: High confidence predictions (tight distribution)

  - Adverse weather: Lower confidence, wider distribution

- Fixed threshold suboptimal:

  - 0.25 too high for foggy conditions -> misses detections

  - 0.25 too low for sunny conditions -> allows false positives

- WACT optimizes precision-recall trade off per condition:

  - Foggy: Lower threshold (0.35) -> prioritize recall

  - Sunny: Higher threshold (0.20) -> prioritize precision

5.2 Augmentation Impact Analysis

What worked:

- HSV augmentations simulate weather color/lighting shifts

- Geometric augmentations handle camera variations

- Mosaic/Mixup improve robustness to partial occlusions

What didn't work as expected:

- Slight degradation in Foggy/Snowy

  - Hypothesis: Over-augmentation may dilute foggy/snowy patterns

  - Baseline model may have "memorized" these specific patterns

Recommendation: Condition specific augmentation strategies

- Heavy augmentation for common conditions (Sunny, Cloudy)

- Light augmentation for rare conditions (Foggy, Snowy)

5.3 Class-Specific Patterns

Distant signals consistently easier to detect:

- mAP@0.5: 66.6% (distant) vs 58.7% (main)

- Recall: 59.5% (distant) vs 49.7% (main)

Possible explanations:

- Distant signals more standardized in appearance

- Main signals have more variation (multi-aspect signals)

- Dataset imbalance (64.7% main vs 35.3% distant)

5.4 Limitations

1. Weather Label Dependency:

- WACT requires weather labels at inference

- Solution: Train weather classifier (ResNet18 on same dataset)

2. Small Test Set:

- Some conditions have <20 test images

- Impact: Lower statistical confidence in results

3. Fixed IoU Threshold:

- Uses IoU=0.5 for all weather

- Improvement: Weather-adaptive IoU thresholds?

4. Single Dataset:

- Only evaluated on GERALD

- Generalization: Need testing on other railway datasets

5.5 Computational Efficiency

Inference Speed:

- Preprocessing: 0.1 ms/image

- Inference: 1.5 ms/image

- Postprocessing: 3.7 ms/image

- Total: ~5.3 ms/image (~189 FPS)

Model Size:

- Weights: 6.2 MB

- Parameters: 3.0M

- Deployment-ready for edge devices

6. Conclusions

6.1 Key Achievements

- Robust Detection System: 62.6% mAP@0.5 on diverse weather conditions

- WACT Method: Systematic weather-adaptive thresholding

- Weather Robustness: Up to 50% improvement in Unknown conditions

- Near-Perfect Foggy Performance: 92.9% F1-score with WACT

- Real-time Capable: 189 FPS inference speed

6.2 Main Contributions

Scientific:

- Weather-Adaptive Confidence Thresholding (WACT) methodology

- Comprehensive weather stratified evaluation protocol

- Empirical analysis of augmentation strategies for weather robustness

Practical:

- Deployable railway signal detection system

- Open-source preprocessing pipeline

- Reproducible training framework

6.3 Future Work

Short-term:

- Automatic weather classification: Eliminate manual weather labelling

- Hybrid approach: Combine YOLO detection with classification refinement

- Extended evaluation: Test on additional railway datasets

Long-term:

- Temporal modeling: Leverage video sequences for tracking

- Multi-task learning: Joint weather prediction + signal detection

- Active learning: Iteratively improve on failure cases

- Deployment study: Real-world testing on railway vehicles

7. Technical Specifications

7.1 Hardware

- GPU: 2× NVIDIA Tesla T4 (15 GB VRAM each)

- RAM: Cached training data (~1.3 GB)

- Training time: 42 minutes (augmented), 24 minutes (baseline)

7.2 Hyperparameters

| Parameter | Value |
| --- | --- |
| Epochs | 50 |
| Batch size | 32 |
| Image size | 640 |
| Optimizer | AdamW |
| Learning rate | 0.001667 |
| Momentum | 0.9 |
| Weight decay | 0.0005 |
| Patience | 15 |
| Workers | 8 |
| Cache | RAM |